🌿 Farm Automation Setup Guide Raspberry Pi • Home Assistant • Sensors

| Who this guide is for This guide assumes you have never used a Raspberry Pi and have no programming or electronics experience. Every step is explained in plain language. If you can follow a recipe you can follow this guide. | What you will end up with - Phone alerts when any water tank drops below 25% - Fence voltage monitored automatically - Soil moisture tracked in specialty plant beds - Weather station logging rainfall and temperature - Cameras watching the shelter and paddock gate - Automatic coop door that locks at dusk every night - A single dashboard on your phone showing everything |
| --- | --- |

| 1 | What is a Raspberry Pi and why do you need one |
| --- | --- |

A Raspberry Pi is a small computer — about the size of a deck of cards — that runs 24 hours a day on about as much power as an LED light bulb. It connects to your home wifi and acts as the brain that links all your sensors and automations together.

You do not need a screen, keyboard or mouse plugged into it after the first setup. Once it is running you access it from your phone or laptop through a web browser, the same way you would check a website.

Home Assistant is free software that runs on the Pi. It gives you a dashboard on your phone showing everything happening on the property, and lets you set up automations — for example, sending you a notification when a water tank gets low, or closing the coop door automatically at sunset.

💡 TIP Think of the Raspberry Pi as the server in your pocket. All your sensors talk to it, it talks to your phone. Once set up you can check on the farm from anywhere with a phone signal.

## What the Pi does NOT do

The Pi is not connected to the internet in a way that exposes your property to hacking — it only talks to devices on your own wifi network and sends outgoing notifications. It does not replace your farm records app — it is a separate system for monitoring and automation only.

| 2 | Everything you need to buy |
| --- | --- |

You do not need to buy everything at once. The Pi kit in Part A is all you need for day one. Add sensors from Parts B and C over the following weeks as you get comfortable.

## Part A — The Raspberry Pi kit (buy first)

| Item | Qty | Each (AUD) | Total (AUD) | Where to buy |
| --- | --- | --- | --- | --- |
| Raspberry Pi 4 Model B — 2GB RAM | 1 | $70–80 | ~$75 | element14.com.au or coreelectronics.com.au |
| Official USB-C power supply 5V 3A | 1 | $12–15 | ~$13 | Core Electronics or Jaycar |
| SanDisk Endurance MicroSD card — 32GB | 1 | $12–15 | ~$13 | JB Hi-Fi, Officeworks, or online |
| MicroSD card reader (USB) | 1 | $5–10 | ~$8 | Officeworks, Jaycar — check if laptop has one first |
| Raspberry Pi case — basic plastic | 1 | $8–12 | ~$10 | Core Electronics or eBay Australia |
| Short ethernet cable — 1m | 1 | $5–8 | ~$6 | Bunnings or Jaycar |
| PART A TOTAL |  |  | ~$125 |  |

⚠ IMPORTANT Do not buy a cheap no-brand power supply. An underpowered supply causes the Pi to crash randomly and is the most common cause of problems for beginners. The official supply is worth the extra few dollars.

## Part B — Sensors for the Pi (buy in first month)

| Item | Qty | Each (AUD) | Total (AUD) | Where to buy |
| --- | --- | --- | --- | --- |
| ESP32 development board (WROOM-32) | 4 | $8–12 | ~$40 | Core Electronics — search "ESP32 DevKit V1" |
| JSN-SR04T waterproof ultrasonic sensor | 2 | $10–14 | ~$24 | Core Electronics or AliExpress (2–4 week wait) |
| Capacitive soil moisture sensor v1.2 | 5 | $8–12 | ~$50 | Core Electronics — NOT the cheap resistive type |
| SHT31 temperature and humidity sensor | 3 | $10–15 | ~$38 | Core Electronics |
| Dupont jumper wires — female to female 20cm pack | 2 | $5–8 | ~$13 | Core Electronics or Jaycar |
| Weatherproof project box — 115x90x55mm | 4 | $8–12 | ~$40 | Jaycar Cat. HB-6003 or similar |
| USB phone chargers — 5V 1A minimum | 4 | $8–12 | ~$40 | Kmart or reject shop — fine for sensors |
| Micro-USB cables — 1.5m | 4 | $5–8 | ~$28 | Kmart or reject shop |
| PART B TOTAL |  |  | ~$273 |  |

💡 TIP AliExpress is roughly half the price of Australian suppliers for ESP32 boards and basic sensors but takes 2–4 weeks to arrive. Order AliExpress items first, then buy from Core Electronics or Jaycar anything you need quickly.

## Part C — Plug-and-play devices (add progressively)

| Item | Qty | Each (AUD) | Total (AUD) | Where to buy |
| --- | --- | --- | --- | --- |
| Ecowitt WS90 weather station | 1 | $120–160 | ~$140 | ecowitt.com or Amazon Australia |
| Reolink E1 Outdoor wifi camera | 2 | $55–70 | ~$120 | reolink.com or JB Hi-Fi |
| Chickcozy automatic coop door opener | 1 | $80–110 | ~$95 | chickcozy.com or Amazon Australia |
| Shelly 1PM power monitor | 1 | $28–35 | ~$32 | shellydomotics.com.au or eBay Australia |
| PART C TOTAL |  |  | ~$387 |  |

Part C devices connect to Home Assistant through wifi — no wiring or soldering required. Add them one at a time when you are ready for each automation.

## Full budget summary

| Part A — Raspberry Pi kit | ~$125 |
| --- | --- |
| Part B — DIY sensors | ~$273 |
| Part C — Plug-and-play devices | ~$387 |
| Total for full setup | ~$785 |

| 3 | Setting up the Raspberry Pi — step by step |
| --- | --- |

This section takes about 30 minutes. You will need your laptop or desktop computer, the Pi, its power supply, the MicroSD card, and the card reader.

## Step 1 — Download Raspberry Pi Imager

- On your laptop, open a web browser and go to: raspberrypi.com/software

- Click the Download button for your operating system (Windows or Mac).

- Open the downloaded file and install it. Accept all the defaults — it is straightforward.

💡 TIP The Imager is free software made by the Raspberry Pi Foundation. It is safe to install and you only need it once.

## Step 2 — Write Home Assistant to the MicroSD card

- Put the MicroSD card into your card reader and plug it into your laptop.

- Open Raspberry Pi Imager.

- Click "Choose Device" and select Raspberry Pi 4.

- Click "Choose OS" then scroll down to "Other specific-purpose OS" and click it. Then click "Home assistants and home automation" then "Home Assistant". Select the most recent version shown.

- Click "Choose Storage" and select your MicroSD card. Make sure you pick the right drive — it will erase everything on it.

- Click "Next". It will ask if you want to apply customisation settings — click "No". For Home Assistant you do not need to customise anything here.

- Click "Yes" to confirm you want to erase the card. The download and write process takes 5–15 minutes depending on your internet speed.

- When it says "Write Successful" you can remove the card.

⚠ IMPORTANT Do not remove the MicroSD card until it says "Write Successful". Removing it early will corrupt the card and you will need to start again.

## Step 3 — First boot

- Put the MicroSD card into the slot on the underside of the Raspberry Pi. It only goes in one way.

- Connect the ethernet cable from the Pi to your wifi router or a network switch. This is faster and more reliable than wifi for the first setup.

- Plug in the power supply. There is no power button — it starts automatically.

- Wait 5 minutes. The first boot takes longer than usual because the system is setting itself up in the background. There is nothing to see during this time — that is normal.

💡 TIP The small green LED on the Pi will flicker during boot. This is normal — it means the SD card is being read. If the green LED stays solid and nothing happens after 10 minutes, unplug, check the card is properly seated, and try again.

## Step 4 — Open Home Assistant

- On any device connected to your home wifi — phone, laptop or tablet — open a web browser.

- Type this address into the browser and press Enter:

homeassistant.local:8123

- If that does not work, try finding your Pi's IP address. Go to your router's admin page (usually 192.168.1.1 or 192.168.0.1 in a browser), log in, and look for a device called "homeassistant" in the connected devices list. Then type that IP address followed by :8123 in the browser.

- The Home Assistant setup wizard will appear. Click "Create my smart home".

- Create a username and password. Write these down and keep them safe — you will need them every time you log in.

- Enter your location. Home Assistant uses this for sunrise and sunset times, which matter for your coop door automation.

- Click through the remaining setup screens. You can skip the optional steps — you will come back to configure everything properly in the next sections.

💡 TIP Bookmark homeassistant.local:8123 on your phone browser and add it to your home screen. Long-press the bookmark in Chrome or Safari and choose "Add to Home Screen" — it will open like an app from then on.

| 4 | Setting up ESPHome — the sensor connection tool |
| --- | --- |

ESPHome is software that runs inside Home Assistant. It is what allows cheap ESP32 microcontrollers to talk to Home Assistant over your wifi network. You configure sensors through simple text files rather than writing code.

You only need to set up ESPHome once. After that, adding new sensors takes about 15 minutes each.

## Installing ESPHome

- In Home Assistant, click "Settings" in the left sidebar.

- Click "Add-ons".

- Click the blue "Add-on Store" button in the bottom right corner.

- In the search box type "ESPHome".

- Click on ESPHome when it appears, then click "Install". This takes 2–5 minutes.

- When installation finishes, turn on all three toggles: "Start on boot", "Watchdog", and "Show in sidebar". Then click "Start".

- ESPHome will now appear in the left sidebar of Home Assistant. Click it to open it.

## What is an ESP32?

The ESP32 is a tiny microcontroller — smaller than a matchbox — with built-in wifi. You connect sensors to it with a few wires, then it sends readings to Home Assistant over your wifi network every few minutes. One ESP32 can handle multiple sensors at once.

You program it once through ESPHome by copying and pasting a configuration file. After that it works automatically forever without needing to be touched again.

💡 TIP The ESP32 is powered by a standard USB phone charger and micro-USB cable. Once configured and mounted in its weatherproof box it needs no maintenance.

| 5 | Build 1 — Water tank level monitor |
| --- | --- |

This is the highest priority build. It monitors your water tank levels and sends a notification to your phone when any tank drops below 25%. Once set up you never need to physically check a tank again.

## What you need for one tank

- 1x ESP32 development board

- 1x JSN-SR04T waterproof ultrasonic distance sensor

- 4x female-to-female dupont jumper wires

- 1x USB phone charger and micro-USB cable

- 1x weatherproof project box

- 1x cable gland or silicone sealant

## How it works

The ultrasonic sensor sits on the lid of your tank pointing straight down at the water surface. It sends a pulse of sound downward and measures how long it takes to bounce back. From that distance it calculates the water level as a percentage and sends the reading to Home Assistant every 2 minutes.

## Wiring — four connections only

No soldering required. The dupont jumper wires push directly onto the pins of both the sensor and the ESP32.

| Sensor pin | → | ESP32 pin | Notes |
| --- | --- | --- | --- |
| VCC (red wire) | → | VIN or 5V pin | Powers the sensor |
| GND (black wire) | → | GND pin | Ground — complete the circuit |
| TRIG (any colour) | → | GPIO 5 | Sends the sound pulse |
| ECHO (any colour) | → | GPIO 18 | Receives the echo |

💡 TIP The GPIO numbers are printed on the ESP32 board next to each pin. Look for the small white text. GPIO 5 and GPIO 18 are usually in the middle section of the board.

## ESPHome configuration — copy and paste this exactly

In ESPHome, click the blue "+ New device" button. Give it a name like "tank-monitor". When it asks for the device type choose ESP32. Click "Skip" when it asks to install — you will paste the configuration first.

Delete everything in the configuration editor and paste the following. Change the values in capitals to match your setup:

esphome: name: tank-monitor esp32: board: esp32dev wifi: ssid: "YOUR_WIFI_NAME" password: "YOUR_WIFI_PASSWORD" api: encryption: key: !secret esphome_key ota: password: !secret esphome_ota sensor: - platform: ultrasonic trigger_pin: GPIO5 echo_pin: GPIO18 name: "Main Tank Level" update_interval: 2min unit_of_measurement: "%" filters: - lambda: ¦- float tank_height = 1.85; float level = (tank_height - x) / tank_height * 100; if (level < 0) return 0; if (level > 100) return 100; return level;

Change tank_height: 1.85 to the actual height of your tank in metres. Measure from where the sensor sits to the very bottom of the tank when empty.

## Installing the firmware onto the ESP32

- Plug the ESP32 into your laptop using the micro-USB cable.

- In ESPHome, click the three dots next to your new device and choose "Install".

- Click "Plug into this computer". Your browser may ask permission to connect to a serial port — click Allow.

- ESPHome will compile the firmware (takes 2–3 minutes) then flash it to the ESP32. You will see a progress log — this is normal.

- When it says "INFO Successfully uploaded" the ESP32 is programmed. Unplug it from the laptop.

- Plug it into a USB phone charger. After 30–60 seconds it should appear as Online in ESPHome.

💡 TIP After the first install, all future updates happen over wifi. You never need to plug the ESP32 into a laptop again unless something goes wrong.

## Mounting the sensor in your tank

- Drill a hole in the lid of your project box the same diameter as the sensor body.

- Push the sensor through from the inside and secure it with the included nut.

- Drill a smaller hole for the power cable and thread it through.

- Put the ESP32 and USB charger inside the box. Connect the jumper wires to the ESP32.

- Seal the cable hole with silicone sealant or a cable gland.

- Drill a hole in the tank lid the same size as the sensor barrel, or mount the box directly above the tank opening.

- Point the sensor straight down toward the water surface. Keep it at least 20cm from the tank wall to avoid false readings.

⚠ IMPORTANT The JSN-SR04T sensor body is waterproof but the circuit board inside the project box is not. Keep the box sealed and mount it above the water line. Condensation inside a poorly sealed box will kill the electronics over time.

| 6 | Build 2 — Soil moisture sensors |
| --- | --- |

These sensors go in the root zone of your specialty plant beds — wasabi, vanilla, finger limes and nasturtiums — and tell you exactly when they need water. Each sensor gives a reading from 0% (bone dry) to 100% (saturated).

## What you need for up to four beds

- 1x ESP32 development board

- 4x capacitive soil moisture sensors v1.2

- Wire to extend sensors from the ESP32 to each bed (2-core or 3-core garden wire works — pick up from Bunnings)

- 1x USB charger and cable

- 1x weatherproof box for the ESP32

⚠ IMPORTANT Buy capacitive sensors only — not resistive. Resistive sensors have two exposed metal probes and corrode within weeks in soil. Capacitive sensors have a coated circuit board and last years. The v1.2 capacitive sensor has "Capacitive Soil Moisture Sensor v1.2" printed on the board.

## Wiring — one ESP32 handles four sensors

| Sensor pin | → | ESP32 pin | Notes |
| --- | --- | --- | --- |
| Sensor 1 AOUT | → | GPIO 32 | Analog signal — soil reading |
| Sensor 1 VCC | → | 3.3V pin | Power |
| Sensor 1 GND | → | GND pin | Ground |
| Sensor 2 AOUT | → | GPIO 33 | Second bed |
| Sensor 2 VCC | → | 3.3V pin | Share the 3.3V pin |
| Sensor 2 GND | → | GND pin | Share the GND pin |
| Sensor 3 AOUT | → | GPIO 34 | Third bed |
| Sensor 4 AOUT | → | GPIO 35 | Fourth bed |

All four sensors share the same VCC and GND pins on the ESP32. You can use a small terminal block to connect multiple wires to a single pin, or twist the wires together and use a lever connector (Wago style) to join them.

## ESPHome configuration for soil moisture

sensor: - platform: adc pin: GPIO32 name: "Wasabi Bed Moisture" update_interval: 5min attenuation: auto unit_of_measurement: "%" filters: - calibrate_linear: - 2.8 -> 0 # Dry reading — calibrate for your sensor - 1.2 -> 100 # Wet reading — calibrate for your sensor - clamp: min_value: 0 max_value: 100 - platform: adc pin: GPIO33 name: "Vanilla Bed Moisture" update_interval: 5min attenuation: auto unit_of_measurement: "%" filters: - calibrate_linear: - 2.8 -> 0 - 1.2 -> 100 - clamp: min_value: 0 max_value: 100 # Repeat pattern for GPIO34 and GPIO35 for remaining beds

## Calibrating the sensors

The calibrate_linear values (2.8 and 1.2 in the example) will be slightly different for each sensor. Here is how to find your exact values:

- Flash the firmware to your ESP32 and let it run with the sensors disconnected from soil.

- Hold a sensor in the air and note the voltage reading that appears in Home Assistant. This is your dry value.

- Hold the sensor fully submerged in a glass of water and note the voltage. This is your wet value.

- Replace 2.8 with your dry value and 1.2 with your wet value in the configuration. Reinstall the firmware (this time it updates over wifi automatically — just click Install in ESPHome).

💡 TIP Different sensors from the same batch can read slightly differently. Calibrating each one individually gives more accurate results. It takes about 5 minutes per sensor.

## Installing sensors in the ground

- Push the sensor into the soil vertically to the base of the board — not just the tip. The entire coated section needs to be in contact with the soil.

- Install it in the active root zone — typically 10–20cm deep depending on the plant.

- Run the wire back to the ESP32 box. Bury the wire or run it through conduit to protect it from the mower and animals.

- For wasabi specifically, install the sensor in the consistently moist zone and set the alert threshold at 60% — wasabi needs to stay consistently damp, not just occasionally watered.

| 7 | Build 3 — Temperature and humidity sensors |
| --- | --- |

These sensors monitor conditions inside your goose shelter and around your specialty plants. One sensor in the shelter alerts you if humidity gets too high — a risk factor for respiratory disease in geese. One near the wasabi bed monitors temperature — wasabi struggles above 25°C.

## Wiring the SHT31 sensor

The SHT31 uses I2C — a two-wire communication protocol that allows multiple sensors on the same two pins. You can run up to four SHT31 sensors on a single ESP32.

| Sensor pin | → | ESP32 pin | Notes |
| --- | --- | --- | --- |
| VCC | → | 3.3V pin | Power — use 3.3V not 5V for SHT31 |
| GND | → | GND pin | Ground |
| SDA | → | GPIO 21 | Data line — same pin for all SHT31 sensors |
| SCL | → | GPIO 22 | Clock line — same pin for all SHT31 sensors |

## ESPHome configuration for temperature and humidity

i2c: sda: GPIO21 scl: GPIO22 scan: true sensor: - platform: sht3xd address: 0x44 temperature: name: "Goose Shelter Temperature" humidity: name: "Goose Shelter Humidity" update_interval: 5min

If you have a second SHT31 sensor, change its address to 0x45 by connecting the ADDR pin to VCC. Then add a second sensor block in the configuration with address: 0x45 and a different name.

| 8 | Plug-and-play devices — no wiring required |
| --- | --- |

These devices connect to Home Assistant through wifi without any wiring or ESP32 boards. They work out of the box.

## Ecowitt weather station

- Mount the weather station in an open area away from buildings and trees — at least 2 metres away from any obstruction. Follow the included instructions for mounting height.

- Power it on. Download the WS View Plus app on your phone and follow the in-app setup to connect it to your wifi.

- In Home Assistant, go to Settings → Devices and Services → Add Integration. Search for "Ecowitt" and click it.

- Follow the prompts. Home Assistant will automatically find the weather station on your network.

- All readings — rainfall, temperature, humidity, wind speed and UV index — will appear as sensors in Home Assistant within a few minutes.

💡 TIP The Ecowitt integration logs all readings to Home Assistant automatically. Your farm records app rainfall log can be updated from the Ecowitt data rather than recording it manually.

## Reolink cameras

- Mount cameras in sheltered positions with a clear view of the goose shelter and paddock gate. They are weatherproof but avoid pointing them into direct afternoon sun — glare makes footage unusable.

- Power them via the included power supply and connect to your wifi using the Reolink app.

- In Home Assistant, go to Settings → Devices and Services → Add Integration. Search for "Reolink" and add your camera IP address.

- The camera feed will appear as a media entity in Home Assistant. You can add it to your dashboard so you can see the shelter directly from your phone.

## Chickcozy automatic coop door

- Mount the door unit on the outside of the goose shelter following the included instructions. The motor pulls a door panel up and down.

- Configure the opening and closing times using the buttons on the unit. Set close time to 15 minutes after your local sunset — you can find this in the Home Assistant dashboard under your location settings.

- The Chickcozy connects to wifi through its own app. Once connected it can be integrated with Home Assistant through the HACS (Home Assistant Community Store) Chickcozy integration — search for setup instructions at community.home-assistant.io.

💡 TIP Even without Home Assistant integration the Chickcozy will automatically close at sunset every night. The Home Assistant integration is optional — it just lets you monitor the door state from your dashboard and get notified if it fails to close.

## Shelly 1PM fence monitor

- The Shelly 1PM is a small module that fits inside a standard power point or inline on a cable. Wire it inline between your power outlet and your fence energiser following the included wiring diagram. Turn off power at the switchboard before doing this.

- Once powered on it creates its own wifi hotspot. Connect to it from your phone and configure your home wifi credentials through the web interface.

- In Home Assistant, go to Settings → Devices and Services → Add Integration. Search for "Shelly" — it will automatically discover the device on your network.

- You will now see the power draw of your energiser in watts as a live sensor. Normal draw for your energiser is around 2–5 watts. A fence short causes the draw to spike — set an automation to notify you when power goes above 8 watts.

| 9 | Setting up automations and notifications |
| --- | --- |

Automations are rules that run automatically — "if this happens, do that". Setting them up in Home Assistant does not require any coding. You create them through a visual editor.

## Setting up phone notifications

- Download the Home Assistant app on your phone. Search "Home Assistant" in the App Store or Google Play — it is free.

- Log in with your Home Assistant address (homeassistant.local:8123) and your username and password.

- The app automatically registers your phone as a notification target. You will see it listed under Settings → Devices and Services → Mobile App.

## Creating an automation — low tank alert

Here is how to create your first automation as an example. The process is the same for all automations.

- In Home Assistant, go to Settings → Automations and Scenes → Create Automation → Create new automation.

- Click "Add Trigger". Choose "Numeric state". Select your tank sensor (e.g. "Main Tank Level"). Set "Below" to 25.

- Skip the Conditions section for now.

- Click "Add Action". Choose "Send a notification via mobile app". Select your phone. Set the title to "Low Tank Alert" and the message to "Main tank is below 25%. Check water supply."

- Click Save. Name the automation "Tank Low Alert".

Your phone will now notify you whenever that tank drops below 25%. Repeat this for each tank sensor you have installed.

## Key automations to set up for your farm

| What to monitor | Trigger condition | Action |
| --- | --- | --- |
| Water tank level | Any tank below 25% | Phone notification |
| Water tank level | Any tank below 10% | Urgent phone notification — check immediately |
| Fence energiser power draw | Watts above 8W for 5+ minutes | Phone notification — possible fence short |
| Goose shelter humidity | Above 80% for 2+ hours overnight | Phone notification — ventilation check needed |
| Wasabi bed temperature | Above 24°C for 1+ hour | Phone notification — consider shade or cooling |
| Wasabi bed moisture | Below 55% | Phone notification — water wasabi bed now |
| Vanilla bed moisture | Below 40% | Phone notification — water vanilla now |
| Coop door | Still open 1 hour after sunset | Urgent notification — check door immediately |
| Daily summary | Every morning at 7am | Notification: rainfall overnight, tank levels, any alerts |

## The daily 7am summary automation

This is the most useful automation. Every morning before you go outside your phone tells you what happened overnight and what needs attention today. Here is how to set it up:

- Create a new automation. Set trigger to "Time" and enter 07:00.

- Add action "Send notification via mobile app". In the message field, click the template icon (the curly brace symbol) and paste the following template:

Good morning. Farm summary for {{ now().strftime("%A %d %B") }}. WATER TANKS Main tank: {{ states("sensor.main_tank_level") ¦ round(0) }}% House tank: {{ states("sensor.house_tank_level") ¦ round(0) }}% WEATHER Rain last 24hrs: {{ states("sensor.ecowitt_rain_day") }}mm Current temp: {{ states("sensor.ecowitt_temperature") }}°C SHELTER Goose shelter humidity: {{ states("sensor.goose_shelter_humidity") ¦ round(0) }}% Wasabi bed moisture: {{ states("sensor.wasabi_bed_moisture") ¦ round(0) }}%

Replace the sensor names in the template with the exact names you gave each sensor in your ESPHome configuration. Sensor names in Home Assistant are formatted as sensor. followed by the name in lowercase with spaces replaced by underscores.

| 10 | Building your farm dashboard |
| --- | --- |

The Home Assistant dashboard is what you see when you open the app on your phone. You can customise it to show exactly what matters for your farm. Here is a suggested layout.

## Adding cards to the dashboard

- In Home Assistant, click on "Overview" in the left sidebar.

- Click the three dots in the top right corner and choose "Edit dashboard".

- Click the blue "+ Add card" button to add your first card.

- Choose the card type from the list that appears.

## Suggested dashboard layout

| Card type | What to show | Settings |
| --- | --- | --- |
| Gauge | Each water tank | Min 0, Max 100, unit %. Set severity: green above 50, yellow 25–50, red below 25 |
| Glance | Soil moisture — all beds | Shows all beds at once as a row of percentages |
| Weather | Ecowitt station | Select your Ecowitt weather entity — shows temperature, rain, wind |
| History graph | Rainfall — 7 days | Set hours to display: 168 (7 days). Good for seeing patterns |
| Thermostat | Shelter temperature | Shows current temp and humidity together |
| Picture glance | Camera feeds | Add your Reolink camera entity — tap to see live view |
| Entity | Coop door status | Shows Open or Closed with a coloured indicator |
| Entity | Fence power draw | Shows watts — you can see at a glance if something is wrong |

| 11 | Troubleshooting common problems |
| --- | --- |

## The Pi is not responding

- Check the power supply is plugged in and the green LED on the Pi is on.

- Try homeassistant.local:8123 in the browser — if it does not load, try waiting another 2 minutes.

- If still nothing, find the Pi's IP address from your router admin page and use that instead.

- If the IP also does not work, unplug the Pi, wait 10 seconds, and plug back in. Wait 5 minutes.

- If it still fails after a reboot, the MicroSD card may be corrupted. Re-flash it using Raspberry Pi Imager and your Pi will restart fresh. Your configuration will be lost but your data in the farm records app is unaffected.

## An ESP32 sensor is showing as Offline in ESPHome

- Check the USB charger is plugged in and the blue LED on the ESP32 is blinking or solid.

- Make sure the ESP32 is in range of your wifi. If it is in a metal shed or far from the router it may need a wifi extender or a longer cable run back to a better location.

- Open ESPHome and click on the device. Click Logs — this shows what the ESP32 is doing and usually explains the problem clearly.

- If the ESP32 lost its wifi connection (e.g. you changed your wifi password) you will need to plug it back into your laptop and re-flash it with the updated wifi credentials.

## A sensor is reading incorrectly

- Soil moisture: check the sensor is fully inserted with all the coated area underground. Recalibrate the dry and wet values as described in Section 6.

- Tank level: measure the actual tank height in metres and make sure the tank_height value in the configuration matches. Also check the sensor is pointing straight down, not at an angle.

- Temperature and humidity: if the reading looks wildly wrong, check the wiring — particularly that SDA is on GPIO21 and SCL is on GPIO22.

## Getting help

The Home Assistant community is large and very helpful for beginners. If you get stuck:

- community.home-assistant.io — the official forum. Search your problem first — it has almost certainly been answered before.

- reddit.com/r/homeassistant — active community, friendly to beginners.

- esphome.io/guides — the ESPHome documentation is excellent and has examples for almost every sensor type.

- YouTube — search "Home Assistant beginner" or "ESPHome [sensor name]" for step-by-step video guides.

| 12 | Quick reference — pins, addresses and settings |
| --- | --- |

## ESP32 pin reference for this guide

| Pin | Used for | Notes |
| --- | --- | --- |
| 3.3V | SHT31 power, soil sensor power | Do not use 5V for these sensors — will damage them |
| 5V / VIN | Ultrasonic sensor power | Tank sensor needs 5V to work correctly |
| GND | Ground — all sensors | Multiple sensors can share GND pins |
| GPIO 5 | Ultrasonic TRIG | Tank sensor — send pulse |
| GPIO 18 | Ultrasonic ECHO | Tank sensor — receive echo |
| GPIO 21 | SHT31 SDA | I2C data — all SHT31 sensors share this pin |
| GPIO 22 | SHT31 SCL | I2C clock — all SHT31 sensors share this pin |
| GPIO 32 | Soil moisture sensor 1 | Analog input — wasabi bed |
| GPIO 33 | Soil moisture sensor 2 | Analog input — vanilla bed |
| GPIO 34 | Soil moisture sensor 3 | Analog input — finger lime bed |
| GPIO 35 | Soil moisture sensor 4 | Analog input — nasturtium / general |

## Suggested alert thresholds

| Sensor | Warning threshold | Urgent threshold | Ideal range |
| --- | --- | --- | --- |
| Water tank level | Below 25% | Below 10% | Above 50% |
| Fence power draw | Above 8W | Above 15W for 10min | 2–5W normal |
| Goose shelter humidity | Above 80% | Above 90% | 50–75% |
| Wasabi bed moisture | Below 55% | Below 40% | 60–80% |
| Vanilla bed moisture | Below 35% | Below 20% | 40–60% |
| Finger lime moisture | Below 30% | Below 15% | 35–55% |
| Wasabi bed temperature | Above 22°C | Above 26°C | 10–18°C ideal |
| Shelter temperature (summer) | Above 38°C | Above 42°C | Below 35°C |

A note on getting started Do not try to do everything at once. Set up the Raspberry Pi and get Home Assistant running. Then add one sensor. Get comfortable with it before adding the next. Within a month you will have a system that saves you more time every single day than it took to set up.